EE230 – HW3 Report
PLL System Design
(Using VerilogA & Matlab)

Muhammad Aldacher

Student ID: 011510317

- Schematic of the PLL system:

Fig. 1. PLL System Block Diagram

Fig. 2. Typical Charge Pump PLL System

Table 1. System Parameters Used

| Parameter | Value |
| --- | --- |
| FREF | 30 MHz |
| FOUT | 1.9 GHz |
| MDivider | 64 |
| ICP | 100 uA |
| KVCO | 600 MHz/V |
| RP | 6.5 KΩ |
| CP | 100 fF |
| C2 | 10 fF |

- System Analysis:

HOpen Loop Gain(s) =  Icp KVCO2πNs R1C1+ 1s2(s R1C1C2 + (C1+ C2))

HClosed Loop Gains =  Icp KVCO2πs R1C1+ 1s3R1C1C2 + s2C1+ C2 + s R1C1Icp KVCO2πN + Icp KVCO2πN

- System Stability (Matlab):

  - Bode Plots:

Fig. 3. Open-Loop Bode Plots

Fig. 4. Closed-Loop Bode Plots

Table 2. Open-Loop & Closed-Loop Parameters

| Parameter |  | Value |
| --- | --- | --- |
| Zero | fz | 0.245 MHz |
| Unity-Gain BW | fugb | 0.871 MHz |
| Pole | fp3 | 2.693 MHz |
| Max Phase Margin | PMMax | 56.44° |
| Phase Margin | PM | 56.38° |
| Closed-Loop BW | BW | 1.41 MHz |

  - Code:

clc; close all;

figure

Icp = 100e-6; 	Kvco = 2*pi*600e6; 	M = 64;

R1 = 6.5e3; 	C1 = 100e-12; 		C2 = C1/10;

%-------------------------------------------------------

% Open Loop Gain:

wz = 1 / (R1*C1)

w_ugb = wz * sqrt((C1/C2)+1);

wp3 = (C1 + C2) / (R1*C1*C2);

PM_Max = (atan(w_ugb/wz) - atan(w_ugb/wp3)) *360/(2*pi)

Const1 = (Icp*Kvco)/(2*pi*M*C1*C2*R1);

H = tf([C1*R1*Const1 Const1],[1 ((C1+C2)/(C1*C2*R1)) 0 0]);

bode(H)

[Gm,Pm,Wgm,Wpm] = margin(H);

Pm

hold on; grid on; title('Loop Gain Transfer function')

set(findall(gcf,'type','line'),'linewidth',2)

%-------------------------------------------------------

% To find the zero & pole accurately:

b = [C1*R1*Const1 Const1];

a = [1 ((C1+C2)/(C1*C2*R1)) 0 0];

fvtool(b,a,'polezero')

[b,a] = eqtflength(b,a);

[z,p,k] = tf2zp(b,a)            %<-- z = zero, p = pole

gain = db2mag(0);

wc = getGainCrossover(H,gain)   %<-- wc = unity-gain BW

%-------------------------------------------------------

% Closed Loop Gain:

figure

H = tf([(C1*R1*Const1*M) (M*Const1)],[1 ((C1+C2)/(C1*C2*R1)) (C1*R1*Const1) Const1]);

bode(H)

[Gm,Pm,Wgm,Wpm] = margin(H);

BW = (bandwidth(H))/(2*pi)

fz = wz / (2*pi)

fp = wp3 / (2*pi)

fc = wc / (2*pi)

hold on; grid on; title('Closed Loop Transfer function')

set(findall(gcf,'type','line'),'linewidth',2)

- System Simulations:

  - Open-Loop Test Bench:

Fig. 5. Test Bench for PFD, CP, LF, & VCO in an Open Loop

Ref

Fb

Up

Dn

Up_bar

Dn_bar

V_control

V_out

Freq_out

Fig. 6. Waveforms when Feedback signal is leading (with respect to Reference)

Ref

Fb

Up

Dn

Up_bar

Dn_bar

V_control

V_out

Freq_out

Fig. 7. Waveforms when Feedback signal is lagging (with respect to Reference)

  - VCO & Divider Test Bench:

Fig. 8. Test Bench for VCO & Divider

F_Div = 29. 6875MHz

F_out = 1.9 GHz

Ref

Fb

Freq_out

Freq_Div

Fig. 9. Output Waveforms of VCO & Divider

  - Whole PLL Test Bench:

Fig. 10. Test Bench for the Whole PLL System

Ref

Fb

Freq_Fb

Up

Dn

Vcontrol

VCO_out

Freq_out

Fig. 11. Output Waveforms of the PLL when locking

- VerilogA codes:

  - PFD code:

// VerilogA for EE230_PLL_VerilogA, PFD, veriloga

`include "constants.vams"

`include "disciplines.vams"

module PFD(up,dn,upb,dnb,fref,fdiv);

parameter real vtrans=0.5;

parameter real delay = 0;

parameter real ttime = 1p;

parameter real clk_threshold = 0.5;

input fref;

input fdiv;

output up,upb,dn,dnb;

electrical fref,fdiv,up,upb,dn,dnb;

real fv_rst, fr_rst, reset, upr,upbr,dnr,dnbr;

analog begin

@(cross(V(fref) - clk_threshold, +1))

begin

upr = 1; upbr = 0; end

@(cross(V(fdiv) - clk_threshold, +1))

begin

dnr = 1; dnbr = 0; end

if(dnr == upr) begin

upr = 0; dnr = 0; upbr = 1; dnbr = 1; end

V(up) <+ transition(upr,delay,ttime); 	V(dn) <+ transition(dnr,delay,ttime);

V(upb) <+ transition(upbr,delay,ttime); 	V(dnb) <+ transition(dnbr,delay,ttime);

end

endmodule

  - Charge Pump code:

// VerilogA for EE230_PLL_VerilogA, CP, veriloga

`include "constants.vams"

`include "disciplines.vams"

module CP(pout,nout,up,dn);

parameter real cur = 100u; //output current 100uA

parameter real threshold=0.5;

input up, dn;

output pout, nout;

electrical pout, nout, up, dn;

real out;

analog begin

@(initial_step) out = 0.5;

if((V(dn)>0.5) && (V(up)<0.5)) begin

out = -cur; end

else if((V(dn)<0.5) && (V(up)>0.5)) begin

out = cur; end

else out = 0.0;

I(pout, nout) <+ -transition(out, 0.0, 10n, 10n);

end

endmodule

  - VCO code:

// VerilogA for testlib, VCO, VerilogA

`include "constants.vams"

`include "disciplines.vams"

module VCO ( vin , out ) ;

input vin ; output out ;

electrical vin ; electrical out ;

parameter real Vmin=0; // Minimum input voltage

parameter real Vmax=Vmin+1 from (Vmin : inf ) ; // Maximum input voltage

parameter real Fmin= 1.6e9 from ( 0 : inf) ; // Minimum output frequency

parameter real Fmax=2.2e9 from (Fmin : inf ) ; // Maximum output frequency

parameter real Vamp = 1 from [ 0 : inf ) ; // Output sinusoid amplitude

parameter real ttol=1u/Fmax from ( 0 : 1 /Fmax ) ; // Crossing time tolerance

parameter real vtol = 1e-9; // Voltage

parameter integer min_pts_update=32 from [ 2 : inf ) ;

parameter real tran_time = 10e-12 from ( 0 : 0.3 /Fmax ) ;

parameter real jitter_std_ui = 0 from [ 0 : 1 ) ;

// Internal Variables

real freq, phase, jitter_rad, dPhase, phase_ideal;

integer n ;

integer seed ;

analog begin

@( initial_step )

begin

seed = 671;

n = 0 ;

dPhase = 0 ;

jitter_rad = jitter_std_ui * 2 * `M_PI ;

end

// compute the freq from the input voltage

freq = ( (V(vin) - Vmin) * (Fmax - Fmin) / (Vmax - Vmin) ) + Fmin ;

$bound_step (1/( min_pts_update * freq ) ) ;

if ( freq > Fmax) freq = Fmax ;

if ( freq < Fmin) freq = Fmin ;

phase_ideal = 2 * `M_PI * idtmod(freq, 0.0, 1.0, -0.5);

phase = phase_ideal + dPhase ;

@( cross(phase_ideal + `M_PI/2, +1, ttol, vtol) or cross(phase_ideal - `M_PI/2, +1, ttol, vtol))

begin

dPhase = $rdist_normal(seed,0,jitter_rad); end

@( cross(phase + `M_PI/2, +1, ttol, vtol) or cross(phase - `M_PI/2, +1, ttol, vtol))

begin

n = ( phase >= -`M_PI/2)&&(phase < `M_PI / 2 ) ; end

// generate the output

V( out ) <+ transition (n?Vamp:0 , 0 , tran_time);

end

endmodule

  - Divider code:

// VerilogA for EE230_PLL_VerilogA, Divider, veriloga

`include "constants.vams"

`include "disciplines.vams"

module Divider(clk,vout);

parameter real vtrans=0.5;

parameter real delay = 0;

parameter real ttime = 1p;

parameter real clk_threshold = 0.5;

parameter real N = 64;

input clk; output vout;

electrical clk,vout;

real counter,v_out;

analog begin

@(initial_step) begin

v_out = 0; counter = 0; end

@(cross(V(clk) - clk_threshold, +1))

begin

if(counter < ((N/2)-1)) begin

counter = counter +1; v_out = 0; end

else if(counter < (N-1)) begin

counter = counter +1; v_out = 1; end

else if (counter == (N-1)) begin

counter = 0; v_out = 0; end

end

V(vout) <+ transition(v_out,delay,ttime);

end

endmodule